Digital Alarm Clock

8051 Assembly Language Code — Explained Simply

Course: EEE 4706  |  Team: C2 Group 1

MCU: AT89S52  |  Crystal: 12 MHz  |  Assembler: ASEM-51

Code Version: v6.0 (Optimized)

What This Document Is This document explains the Digital Alarm Clock assembly code in plain English. You do not need to be an expert programmer to understand it. Each section of the code is broken down step by step with simple analogies and diagrams so anyone can follow along.

# 1. The Big Picture — What Does This Code Do?

Imagine a wristwatch combined with three alarm clocks and a snooze button. That is exactly what this code builds on a tiny microcontroller chip. Here is everything the clock does:

- Displays the current time (HH:MM:SS) on a 16×2 LCD screen

- Lets you set the current time using the keypad

- Lets you set up to 3 independent alarms

- Sounds a buzzer and flashes an LED when an alarm fires

- Lets you snooze an alarm for a custom number of minutes

- Stops an alarm permanently by pressing D, or by entering the secret sequence 1→2→1

The code is written in 8051 Assembly Language — a very low-level language where every single instruction talks directly to the hardware.

## 1.1  Hardware Connected to the MCU

| Pin / Port | What Is Connected There |
| --- | --- |
| P1.0 – P1.7 | LCD data bus (8 wires carrying the characters to display) |
| P2.0 (LCD_RS) | LCD Register Select — tells LCD if we are sending a command or data |
| P2.1 (LCD_RW) | LCD Read/Write — always set to Write (LOW) in this code |
| P2.2 (LCD_EN) | LCD Enable — a pulse on this pin makes the LCD accept the data |
| P2.3 (BUZZER) | Active-LOW buzzer — CLR = buzzer ON, SETB = buzzer OFF |
| P2.4 (LED1) | Red LED — flashes together with the buzzer during alarm |
| P2.5 (LED2) | Green LED — power/status indicator, stays ON normally |
| P3.0 – P3.3 | Keypad rows (KR1–KR4) — driven LOW one at a time to scan keys |
| P3.4 – P3.7 | Keypad columns — read to detect which key is pressed |

# 2. Memory — Where Does the Code Store Data?

The 8051 has a small block of internal RAM (Random Access Memory) where it keeps all its working data. Think of it as a row of labelled boxes, each holding one number. Here is what each box stores:

## 2.1  Time Variables (addresses 30H – 32H)

| Variable | What It Stores |
| --- | --- |
| SEC (30H) | Current seconds (0 – 59) |
| MINR (31H) | Current minutes (0 – 59) |
| HOUR (32H) | Current hours (0 – 23) |

## 2.2  Alarm Variables (addresses 33H – 3BH)

Three alarms, each needing three values: the hour to ring, the minute to ring, and whether it is switched on (enabled).

| Variable | What It Stores |
| --- | --- |
| A1HR (33H) | Alarm 1 — hour to ring |
| A1MN (34H) | Alarm 1 — minute to ring |
| A1EN (35H) | Alarm 1 — enabled? (01H = yes, 00H = no) |
| A2HR / A2MN / A2EN | Same three values for Alarm 2 |
| A3HR / A3MN / A3EN | Same three values for Alarm 3 |

## 2.3  Snooze Variables (addresses 3DH – 40H)

| Variable | What It Stores |
| --- | --- |
| SNZMIN (3DH) | How many minutes the snooze lasts |
| SNZACT (3EH) | Is snooze active? (01H = yes, 00H = no) |
| SNZHR (3FH) | The hour when the snooze alarm will fire again |
| SNZMN (40H) | The minute when the snooze alarm will fire again |

## 2.4  Delay Counters (addresses 42H – 44H)

These three memory locations are used exclusively as loop counters inside the delay routines. They are kept separate to avoid a bug where one delay accidentally overwrites another delay's counter.

| Variable | Used By |
| --- | --- |
| DC1 (42H) | DLY_1MS only — the innermost delay loop |
| DC2 (43H) | DLY_5MS, DLY_10MS, DLY_20MS — the middle layer |
| DC3 (44H) | DLY_100MS, DLY_1S — the outermost layer |

## 2.5  Temporary / General Variables (addresses 50H – 53H)

| Variable | What It Stores |
| --- | --- |
| V1 (50H) | The last key pressed (ASCII character), or the result of GET_2DIG |
| V2 (51H) | Temporary — holds the tens digit while reading a 2-digit number |
| V3 (52H) | Result of the button-sequence checker: 01H = sequence complete |
| VN (53H) | Which alarm is currently active (0=snooze, 1, 2, or 3) |

# 3. Startup — How Does the Chip Know Where to Begin?

When the AT89S52 is powered on or reset, it always starts executing from address 0000H in flash memory. The very first instruction it finds there is:

ORG 0000H

LJMP MAIN          ; jump to the MAIN routine

This is like the front door of the program. The chip walks in, immediately reads 'go to MAIN', and jumps there.

A second important address is 000BH — this is the Timer 0 interrupt vector. When Timer 0 overflows and triggers an interrupt, the chip automatically jumps to 000BH:

ORG 000BH

LJMP T0_ISR        ; jump to Timer 0 Interrupt Service Routine

What is an Interrupt? An interrupt is like a tap on the shoulder. The CPU is busy running the main loop, but every 10 milliseconds Timer 0 taps it on the shoulder and says 'stop what you are doing, update the clock'. The CPU saves its place, runs T0_ISR, then returns to exactly where it left off.

# 4. The Clock Engine — Timer 0 ISR

This is the heart of the timekeeping system. It fires exactly every 10 milliseconds, counts up to 100 ticks (= 1 second), then increments the seconds, minutes, and hours.

## 4.1  How Is 10ms Achieved?

The AT89S52 uses a 12 MHz crystal. At 12 MHz, one machine cycle takes exactly 1 microsecond (1 µs). Timer 0 counts machine cycles and overflows after a preset number of counts.

| Calculation Step | Value |
| --- | --- |
| Target period | 10 ms = 10,000 µs |
| Counts needed | 10,000 counts (at 1 µs each) |
| Timer max value | 65,536 (16-bit timer) |
| Reload value | 65,536 − 10,000 = 55,536 |
| In hexadecimal | 55,536 = 0xD8F0 |
| TH0 (high byte) | 0xD8 |
| TL0 (low byte) | 0xF0 |

Every time the ISR fires, it immediately reloads TH0 and TL0 so the next 10ms period starts instantly:

MOV  TH0, #0D8H

MOV  TL0, #0F0H

## 4.2  Counting Up to One Second

The variable TICK counts how many 10ms interrupts have occurred. When it reaches 100, one full second has passed:

INC  TICK

MOV  A, TICK

CJNE A, #100, T0_EXIT   ; not 1 second yet, leave

MOV  TICK, #00H         ; reset tick counter

INC  SEC                ; add one second

## 4.3  Rollover Chain

The ISR handles all rollovers automatically — seconds roll over at 60, minutes at 60, hours at 24, creating a proper 24-hour clock:

Rollover Logic (simplified) TICK reaches 100 → TICK resets to 0, SEC increases by 1 SEC reaches 60 → SEC resets to 0, MINR increases by 1 MINR reaches 60 → MINR resets to 0, HOUR increases by 1 HOUR reaches 24 → HOUR resets to 0 (midnight rollover)

# 5. MAIN — Startup and Initialisation

MAIN runs once at power-on. It prepares everything before the clock starts running. Think of it as setting up the room before a show begins.

| What MAIN Does | Why |
| --- | --- |
| MOV SP, #6FH | Sets the stack pointer — tells the CPU where to save return addresses when calling subroutines |
| MOV SEC/MINR/HOUR, #00H | Initialises time to 00:00:00 |
| MOV A1EN/A2EN/A3EN, #00H | Disables all three alarms at startup |
| MOV SNZMIN, #05H | Default snooze duration = 5 minutes |
| SETB BUZZER | Buzzer OFF (active-LOW: SETB = logic 1 = OFF) |
| CLR LED1 / SETB LED2 | Red LED off, Green status LED on |
| MOV TMOD, #01H | Configures Timer 0 in Mode 1 (16-bit timer) |
| MOV TH0/TL0 | Loads the 10ms reload value (0xD8F0) |
| SETB ET0 / SETB EA / SETB TR0 | Enables Timer 0 interrupt, enables global interrupts, starts the timer |
| LCALL LCD_INIT | Sends the HD44780 initialisation sequence to the LCD |
| LCALL SHOW_MENU | Displays the menu on the LCD |

# 6. The Main Loop — MLOOP

After MAIN finishes, the program enters MLOOP — an infinite loop that runs forever. Every iteration of the loop does four things:

- Updates the LCD with the current time (DISP_TIME)

- Checks if any alarm should fire (CHK_ALARMS)

- Scans the keypad for a key press (CHK_KEY_NB)

- Waits 100ms before repeating (DLY_100MS)

This means the display refreshes and alarms are checked 10 times per second — fast enough to feel instant to a human but not so fast that it wastes power.

## 6.1  Key Handling in the Main Loop

When a key is pressed, the code identifies which key it was and acts accordingly:

| Key Pressed | What Happens |
| --- | --- |
| 1 | Calls MENU_SET_TIME — lets you type in a new current time |
| 2 | Sets VN=1 then calls MENU_SET_ALM — sets Alarm 1 |
| 3 | Sets VN=2 then calls MENU_SET_ALM — sets Alarm 2 |
| 4 | Sets VN=3 then calls MENU_SET_ALM — sets Alarm 3 |
| Any other key | Ignored — loop just continues |

## 6.2  Why CHK_KEY_NB Instead of GET_KEY?

CHK_KEY_NB (Non-Blocking) checks whether any key is currently pressed without waiting. If no key is pressed it returns immediately with V1 = 00H. This is important because the loop must keep checking alarms — it cannot afford to get stuck waiting for a keypress that may never come.

# 7. CHK_ALARMS — Checking if an Alarm Should Fire

CHK_ALARMS runs on every loop iteration. It compares the current time against each alarm's programmed time. If they match AND the alarm is enabled AND seconds = 00, the alarm fires.

The snooze alarm is checked first, then Alarm 1, 2, and 3 in order. The checking logic for each alarm is:

Is this alarm enabled?          No → skip to next alarm

Does HOUR  match alarm hour?    No → skip to next alarm

Does MINR  match alarm minute?  No → skip to next alarm

Is  SEC  == 00?                 No → skip to next alarm

All matched → disable alarm, call ALM_ROUTINE

Requiring SEC = 00 ensures the alarm only fires once per minute at the exact start of that minute, not every second throughout the minute.

# 8. ALM_ROUTINE — The Alarm Handler

When an alarm fires, ALM_ROUTINE takes over completely. The main loop is paused until the alarm is dealt with. This routine handles Extra Feature 1 (LED flash) and Extra Feature 2 (sequence stop).

## 8.1  The Alarm Cycle

ALM_ROUTINE loops repeatedly in a beep-pause-beep-pause pattern:

One alarm cycle (600ms total) Step 1: Update LCD — show '!! ALARM N' and the hint '1>2>1 A=Snz D=X' Step 2: CLR BUZZER (ON) + SETB LED1 (ON) — beep and flash for 300ms During these 300ms: scan keypad every 10ms (ALM_POLL) Step 3: SETB BUZZER (OFF) + CLR LED1 (OFF) — silent for 300ms During these 300ms: scan keypad every 10ms (ALM_POLL) Step 4: If no key pressed in either phase, repeat from Step 1 If a key was pressed, jump to ALM_GOTKEY

## 8.2  ALM_POLL — The Smart Key Scanner

ALM_POLL is a sub-routine that scans the keypad 30 times over 300 milliseconds (one scan every 10ms). The moment it detects any key, it returns immediately with that key in V1. If no key is pressed after all 30 scans, it returns with V1 = 00H.

This gives a full 600ms detection window every alarm cycle, so the user can press a key at any natural moment without needing perfect timing.

## 8.3  What Happens When a Key Is Pressed

| Key | Action |
| --- | --- |
| A | Calls DO_SNOOZE — asks for snooze duration, schedules a future re-alarm |
| D | Shows 'Alarm Stopped!' and returns — permanently stops this alarm |
| 1 → 2 → 1 | Sequence stop (Extra Feature 2) — stops alarm with confirmation message |
| Any other key | Passed to CHK_SEQ — advances or resets the 1→2→1 sequence tracker |

# 9. CHK_SEQ — The Button Sequence Checker

This routine tracks whether the user has pressed 1, then 2, then 1 in the correct order. It uses SEQIDX (0, 1, or 2) to remember how far through the sequence the user is.

| SEQIDX value | What it means / What key is expected next |
| --- | --- |
| 0 | At the start — waiting for the first '1' |
| 1 | Got the first '1' — waiting for '2' |
| 2 | Got '1' then '2' — waiting for the final '1' |

If the user presses the right key: SEQIDX advances. If the wrong key is pressed: SEQIDX resets to 0 (unless the wrong key happened to be '1', in which case SEQIDX goes to 1 since that could be the start of a fresh attempt).

When SEQIDX reaches 2 and the user presses '1': V3 is set to 01H (sequence complete!) and SEQIDX resets.

# 10. DO_SNOOZE — Snooze Function

When the user presses A during an alarm, DO_SNOOZE is called. It:

- Turns off the buzzer and LED immediately

- Displays 'Snooze Duration' and 'Mins(01-59):' on the LCD

- Waits for the user to type a 2-digit number of minutes

- Calculates the wakeup time: current MINR + entered minutes

- Handles minute overflow (e.g. 55 + 10 = 65 → becomes 05 with SNZHR + 1)

- Sets SNZACT = 01H to mark snooze as active

When the snooze time arrives, CHK_ALARMS detects SNZACT=01H and triggers ALM_ROUTINE again, just like a normal alarm.

# 11. MENU_SET_TIME and MENU_SET_ALM

## 11.1  MENU_SET_TIME

Called when the user presses '1'. Sequence:

- Call KEY_RELEASE — wait for the '1' key to be released so it is not accidentally read as the first digit

- Display '--- Set Time --' on LCD line 1

- Display 'Hour(00-23):' on LCD line 2

- Call GET_2DIG — wait for two digits, return value in V1

- Store V1 into HOUR

- Display 'Min (00-59):' on line 2

- Call GET_2DIG again — store result into MINR

- Reset SEC to 00

- Show 'Time Set OK!' for 1 second, then return

## 11.2  MENU_SET_ALM

Called when the user presses 2, 3, or 4 (with VN already set to 1, 2, or 3). It uses the same flow as MENU_SET_TIME but stores the result into the correct alarm's HR, MN, and EN variables. After storing, it sets EN = 01H to enable the alarm.

# 12. DISP_TIME — Displaying the Clock

DISP_TIME writes the current time and alarm status to the LCD every loop cycle. It does not clear the screen first — it overwrites in place to avoid flicker.

Line 1 always shows:

Time:HH:MM:SS

Line 2 shows which alarms are armed:

A:1--    (Alarm 1 on, Alarms 2 and 3 off)

A:123    (All three alarms armed)

A:---    (No alarms set)

The P2D sub-routine is used to print any number as exactly 2 decimal digits (e.g. 7 becomes '07'). It works by dividing by 10 — the quotient is the tens digit, the remainder is the units digit — then adding 30H to convert each to its ASCII character code.

# 13. Keypad Routines

## 13.1  How the Matrix Keypad Works

A 4×4 matrix keypad has 4 row wires and 4 column wires. When you press a key, it connects one row wire to one column wire. The MCU finds which key was pressed by:

- Driving Row 1 LOW (all others HIGH), then reading all 4 column pins

- If a column reads LOW → that key at Row1 × that column is pressed

- Otherwise, drive Row 2 LOW and repeat

- Continue for Row 3 and Row 4

## 13.2  GET_KEY — Blocking Key Read

GET_KEY scans all four rows in a loop, waiting forever until a key is pressed. Once a key is detected:

- It waits 20ms (DLY_20MS) for the mechanical bounce to settle

- It re-reads the column pins to confirm which key is truly pressed

- It stores the corresponding ASCII character in V1

- It calls KEY_RELEASE to wait until the finger lifts off

- Then it returns

## 13.3  CHK_KEY_NB — Non-Blocking Key Check

CHK_KEY_NB does a single quick scan of all rows. If any column reads LOW, a key is down and it calls GET_KEY to identify it. If no key is down, it returns immediately with V1 = 00H.

## 13.4  KEY_RELEASE — Preventing Key Bleed

KEY_RELEASE drives all rows LOW simultaneously and monitors the column pins. It loops until all columns read HIGH again — meaning no key is pressed. Then it waits an extra 20ms for the contacts to fully settle.

This prevents a common bug where pressing '2' to set an alarm accidentally also gets read as the first digit of the alarm hour input.

# 14. LCD Driver

## 14.1  LCD_INIT — Initialisation Sequence

The LCD uses the industry-standard HD44780 controller. It must receive a specific sequence of commands when first powered on, with carefully timed delays between them. This is the standard power-on sequence from the HD44780 datasheet:

| Command Sent | What It Does |
| --- | --- |
| #30H (×3 with delays) | Wake-up pulses — required by HD44780 spec |
| #38H | 8-bit mode, 2 display lines, 5×8 dot font |
| #08H | Display OFF (while finishing setup) |
| #01H | Clear display |
| #06H | Entry mode: cursor moves right, display does not shift |
| #0CH | Display ON, cursor OFF, blink OFF |

## 14.2  LCD_CMD and LCD_WDAT

Every piece of data sent to the LCD uses the same pulse sequence:

- Put the byte on P1 (the data bus)

- Set RS appropriately (LOW for commands, HIGH for character data)

- Set RW LOW (always writing)

- Pulse EN HIGH then LOW — this is the strobe that makes the LCD accept the data

- Wait for the LCD controller to process it

## 14.3  LCD_CH — Clear and Home

LCD_CH is a helper that sends 'Clear Display' (#01H) followed by 'Set cursor to position 0,0' (#80H). It is called every time a menu or message needs a fresh screen.

## 14.4  LCD_PSTR — Print a String

LCD_PSTR reads characters one by one from code memory using MOVC (a special instruction that reads from flash rather than RAM), sends each to LCD_WDAT, advances the DPTR pointer, and stops when it finds a null byte (00H) which marks the end of the string.

# 15. Delay Routines

All delays are software delays — the CPU simply runs a tight loop doing nothing useful for a calculated number of iterations. At 12 MHz, each DJNZ instruction takes 2 µs (2 machine cycles × 1 µs each).

## 15.1  The Nesting Problem and Its Fix

A classic bug in 8051 delay routines: if DLY_5MS uses R7 as its loop counter and then calls DLY_1MS which also uses R7, DLY_1MS overwrites R7 before DLY_5MS can finish counting. The solution here is three dedicated RAM counter variables:

| Counter | Used By (exclusively) |
| --- | --- |
| DC1 | DLY_1MS — the innermost loop |
| DC2 | DLY_5MS, DLY_10MS, DLY_20MS — the middle level |
| DC3 | DLY_100MS, DLY_1S — the outermost level |

## 15.2  Delay Calculation for 12 MHz

| Delay | How It Is Calculated |
| --- | --- |
| DLY_1MS | 2 × 250 iterations × 2 µs = 1,000 µs = 1 ms |
| DLY_5MS | 5 × DLY_1MS = 5 ms |
| DLY_10MS | 10 × DLY_1MS = 10 ms |
| DLY_20MS | 20 × DLY_1MS = 20 ms |
| DLY_100MS | 20 × DLY_5MS = 100 ms |
| DLY_1S | 200 × DLY_5MS = 1,000 ms = 1 second |

# 16. GET_2DIG — Reading a Two-Digit Number

GET_2DIG is used whenever the user needs to type a number with exactly two digits — hours, minutes, or snooze duration. It works in two steps:

- Step 1 (Tens digit): Calls GET_KEY, echoes the character to the LCD, converts from ASCII to binary (subtract 30H), stores in V2

- Step 2 (Units digit): Calls GET_KEY again, echoes it, converts to binary, stores in V1

- Step 3 (Combine): V1 = V2 × 10 + V1   (uses the 8051 MUL instruction)

Example: User presses '0' then '9'

Tens digit:  '0' → 30H - 30H = 0   → V2 = 0

Units digit: '9' → 39H - 30H = 9   → V1 = 9

Combine:     0 × 10 + 9 = 9         → V1 = 9 (decimal nine)

# 17. String Constants

All text displayed on the LCD is stored as null-terminated strings in flash (code) memory. Each string ends with a 00H byte which LCD_PSTR uses to know when to stop printing.

| String Label | Text Displayed |
| --- | --- |
| MSG_M1 | 1:Time 2:Alm1 |
| MSG_M2 | 3:Alm2 4:Alm3 |
| MSG_TL | Time: |
| MSG_ALM | !! ALARM |
| MSG_HINT | 1>2>1 A=Snz D=X |
| MSG_STOP | Alarm Stopped! (shared by D-stop and sequence-stop) |
| MSG_SOK | Seq Correct :) |
| MSG_SN1 | Snooze Duration |
| MSG_SN2 | Mins(01-59): |
| MSG_SND | Snoozed! |
| MSG_STIM | --- Set Time -- |
| MSG_EHR | Hour(00-23): |
| MSG_EMN | Min (00-59): |
| MSG_TST | Time Set OK! |
| MSG_SALM | Set Alarm # |
| MSG_ASET | Alarm |
| MSG_SETOK | Set OK! |

# 18. Program Flow Summary

Here is the complete flow of the program from power-on to steady-state operation:

Program Flow (Power ON to Running) 1. Power on / Reset → CPU jumps to address 0000H 2. MAIN runs: initialise all variables, configure Timer 0, start interrupts, init LCD 3. SHOW_MENU displays the keypad menu on the LCD 4. MLOOP starts — this runs forever: a. DISP_TIME — update LCD with HH:MM:SS and alarm status b. CHK_ALARMS — check snooze and all 3 alarms against current time If match found → call ALM_ROUTINE (alarm takes over until resolved) c. CHK_KEY_NB — non-blocking keypad scan Key '1' → MENU_SET_TIME → SHOW_MENU Key '2' → MENU_SET_ALM (Alarm 1) → SHOW_MENU Key '3' → MENU_SET_ALM (Alarm 2) → SHOW_MENU Key '4' → MENU_SET_ALM (Alarm 3) → SHOW_MENU No key → continue d. DLY_100MS — wait 100ms, then repeat from step 4a Meanwhile (running in background, every 10ms): Timer 0 fires T0_ISR → TICK++ → if 100: SEC++ → if 60: MINR++ → if 60: HOUR++

ALM_ROUTINE Flow (when alarm fires) 1. Reset SEQIDX to 0 2. Display alarm screen on LCD 3. CLR BUZZER (ON) + SETB LED1 — poll keypad for 300ms 4. SETB BUZZER (OFF) + CLR LED1 — poll keypad for 300ms 5. If no key in 600ms → go back to step 2 6. Key pressed: 'A' → DO_SNOOZE (get duration, schedule re-alarm, return) 'D' → Show 'Alarm Stopped!', return Other → CHK_SEQ (track 1→2→1 sequence) Sequence complete → Show 'Alarm Stopped! / Seq Correct', return Wrong key → go back to step 2

# 19. Key Assembly Concepts Used in This Code

| Assembly Concept | Plain English Explanation |
| --- | --- |
| BIT directive | Defines a name for a single bit inside an SFR — e.g. LCD_RS BIT P2.0 means P2.0 can now be referred to as LCD_RS |
| DATA directive | Declares a RAM address as a named variable — tells the assembler 'SEC means address 30H in internal RAM' |
| ORG | Sets the address where the next instruction will be placed in flash memory |
| LJMP / SJMP | Long Jump (3 bytes, any address) and Short Jump (2 bytes, ±127 bytes) |
| LCALL / RET | Long Call — saves the return address on the stack, then jumps. RET pops the return address and jumps back |
| CJNE A, #val, label | Compare and Jump if Not Equal — the workhorse comparison instruction |
| DJNZ reg, label | Decrement and Jump if Not Zero — used for all delay loops |
| PUSH ACC / POP ACC | Save and restore the Accumulator on the stack — essential inside ISRs |
| MOVC A, @A+DPTR | Read a byte from code (flash) memory — used for string and key table lookup |
| MUL AB | Multiply A × B — result is in A (low byte) and B (high byte). Used in GET_2DIG |
| DIV AB | Divide A / B — quotient in A, remainder in B. Used in P2D to split digits |
| CLR C / SUBB A, #val | Clear Carry flag, then subtract with carry — CLR C must come first or the subtraction is wrong |
| RETI | Return from Interrupt — like RET but also re-enables the interrupt system |
| SETB EA / ET0 / TR0 | Enable global interrupts, enable Timer 0 interrupt, start Timer 0 running |

# 20. Quick Reference — How to Use the Clock

| What You Want to Do | Keys to Press |
| --- | --- |
| Set current time | Press 1 → type hour (2 digits) → type minute (2 digits) |
| Set Alarm 1 | Press 2 → type hour → type minute |
| Set Alarm 2 | Press 3 → type hour → type minute |
| Set Alarm 3 | Press 4 → type hour → type minute |
| Snooze when alarm rings | Press A → type number of minutes (e.g. 05 for 5 minutes) |
| Force stop alarm | Press D |
| Sequence stop alarm | Press 1, then 2, then 1 (in order) |
| Check which alarms are on | Look at line 2 of LCD: A:1-- means only Alarm 1 is set |

End of Document — Digital Alarm Clock Code Explanation v6.0